From: https://chatgpt.com/c/69fcbe8e-dddc-83ea-9085-839f27420327

# you asked: New-to-us

message time: 2026-05-07 12:32:20

New-to-us server (Relion xe4112 with an lsi 3108 and 36 drive bays). Some of the drives were made before 2020, I'm concerned about their health/how much abuse they've seen. Tell me how to investigate drive health, what to look out for (thesholds and confidence levels), how to identify marginal drives, etc.

# chatgpt response: I’ll frame

I’ll frame this like an acceptance test for inherited storage: first identify exactly what the controller can see, then separate “hard fail” drives from “marginal” drives, then burn them in under controlled monitoring so weak units expose themselves before production.

One important bias: don’t treat “SMART overall-health PASSED” or “array Optimal” as enough. On inherited enterprise disks, the useful signal is usually error trend, controller event history, slot-to-slot outliers, and behavior during full-surface reads/writes.

## Bottom line

For this server, I would not decide drive health from a single SMART pass/fail result. I’d build a baseline for every slot/serial, run full-surface stress before production, then compare counter deltas. A 6+ year-old disk with old static counters may be usable as a spare/lab disk. A disk that creates new media errors, pending sectors, uncorrectables, timeouts, resets, or PHY errors during burn-in is not production-worthy.

SMART is useful, but imperfect. Seagate notes that SMART errors are near-term failure predictions, that some tests can still pass even when SMART is warning, and that the actual vendor thresholds differ by design and are not fully public. (Seagate.com) Seagate’s openSeaChest documentation makes the same point in a more engineering-oriented way: SMART attributes are vendor-specific, raw values are not safely comparable across different products, and the formal “return SMART status” result is basically pass/fail. (GitHub) Backblaze’s large-drive dataset is useful as a sanity check: they specifically investigate SMART 5, 187, 188, 197, and 198 when raw values are greater than zero, but they also found that about 23.3% of failed drives showed no warning from those SMART stats. (Backblaze)

# 1. First collect a clean controller inventory

On an LSI/Broadcom MegaRAID controller, use StorCLI as the controller source of truth. StorCLI is the successor to MegaCLI for LSI MegaRAID controllers, and the common addressing format is /cX for controller, /eX for enclosure, and /sX for slot. (Thomas-Krenn.AG) IBM’s StorCLI reference also lists the same core commands: storcli show, storcli show all, and storcli /cx[/eall]/sall show for physical drives. (IBM)

Run this before creating final arrays:

bash

mkdir -p /root/drive-audit/$(date +%F_%H%M)

cd /root/drive-audit/$(date +%F_%H%M)


      

storcli /c0 show all              > c0_show_all.txt

storcli /c0/eall/sall show        > pd_summary.txt

storcli /c0/eall/sall show all    > pd_all.txt

storcli /c0/vall show all         > vd_all.txt

storcli /c0/fall show             > foreign.txt

storcli /c0 show events           > events.txt

storcli /c0 show termlog          > termlog.txt 2>&1 || true

storcli /c0/eall/sall show phyerrorcounters > phyerrorcounters.txt 2>&1 || true

Broadcom’s current StorCLI docs also include controller commands for drive predictive failure events, including storcli /c0 show pdfailevents, so try that too if your installed StorCLI supports it. (TechDocs)

bash

storcli /c0 show pdfailevents > predictive_failure_events.txt 2>&1 || true

What you want from this stage:

text

Controller firmware

BIOS/FW package version

BBU/CVPM health

Drive state: Onln / UGood / GHS / DHS / UBad / Offln / Frgn

EID:Slt

DID

Serial number

Model

Firmware

Interface: SAS/SATA

Sector size: 512n / 512e / 4Kn

Media Error Count

Other Error Count

Predictive Failure Count

Shield Counter

Temperature

Link speed

PHY error counters

The most important thing is to build a table like this:

text

Bay  EID:Slt  DID  Serial        Model        FW     Size   State   Media  Other  PredFail  Shield  Temp

00   252:0    17   Z840ABC1      ST8000NM...  SN05   8TB    UGood   0      0      0         0       33C

01   252:1    18   Z840ABC2      ST8000NM...  SN05   8TB    UGood   0      7      0         0       34C

...

If you do not preserve slot + serial + DID, you will lose the ability to tell whether a problem follows the drive or follows the bay/backplane path.

# 2. Pull SMART/health directly from each physical drive

smartctl can query disks behind MegaRAID using -d megaraid,N, where N is the MegaRAID disk index, and the smartmontools manual gives Linux examples like smartctl -a -d megaraid,2 /dev/sda. (GitHub)

Usually, /dev/sda can be any logical disk exposed by that controller. The megaraid,N number is the important part.

Example:

bash

# List candidate MegaRAID DIDs from StorCLI output first.

# Then query each one:

for did in $(storcli /c0/eall/sall show | awk '/^[0-9]+:[0-9]+/ {print $3}' | sort -n); do

  echo "=== DID $did ==="

  smartctl -x -d megaraid,$did /dev/sda > smart_did_${did}.txt 2>&1

done

If that fails because the controller/driver path is weird, try /dev/bus/0 or another block device exposed by the controller:

bash

smartctl -x -d megaraid,17 /dev/sda

smartctl -x -d megaraid,17 /dev/bus/0

For SAS drives, do not expect the same pretty ATA SMART table you get from SATA disks. SAS health is often in SCSI log pages and may show things like:

text

SMART Health Status

Elements in grown defect list

Error counter log:

  read:  total corrected errors, total uncorrected errors

  write: total corrected errors, total uncorrected errors

  verify: total corrected errors, total uncorrected errors

Non-medium error count

Self-test log

Background scan results

For SATA disks, watch the classic attributes:

text

5    Reallocated_Sector_Ct

187  Reported_Uncorrect

188  Command_Timeout

197  Current_Pending_Sector

198  Offline_Uncorrectable

199  UDMA_CRC_Error_Count

194  Temperature_Celsius

9    Power_On_Hours

Backblaze specifically flags 5, 187, 188, 197, and 198 as the SMART attributes they use for failure investigation when raw values are greater than zero. (Backblaze)

# 3. My practical thresholds

These are not vendor warranty thresholds. These are production acceptance thresholds for an old inherited storage chassis where you care about IOPS, resiliency, and not inheriting someone else’s abused disks.

## Immediate reject / do not use in production

| Signal | Threshold | Confidence |
| --- | --- | --- |
| Controller predictive failure | Any nonzero predictive failure count | Very high |
| SMART overall-health failed | Any fail / prefail result | Very high |
| New media errors during testing | Any increase | Very high |
| New uncorrectable read/write/verify errors | Any increase | Very high |
| SMART 197 current pending sectors | >0 on SATA | Very high |
| SMART 198 offline uncorrectable | >0 on SATA | Very high |
| SMART 187 reported uncorrectable | >0 on SATA | High to very high |
| Reallocated sectors increasing | Any increase | High |
| SAS grown defect list increasing | Any increase | High |
| Drive drops offline / resets / disappears | Any occurrence under burn-in | Very high |
| Long SMART self-test fails | Any failure | Very high |
| Consistency check finds uncorrectable error | Any occurrence | Very high |
| Repeated command timeouts | More than one, or any increase during burn-in | High |
| Shield counter increments | Any repeat occurrence | High |
| Kernel I/O errors | Any on a candidate production disk | Very high |

For your use case, I’d be ruthless. A disk with new badness during burn-in is not “maybe okay.” It is a future outage volunteering early.

## Marginal / quarantine / maybe spare only

| Signal | Threshold | Confidence |
| --- | --- | --- |
| Static reallocated sectors | 1–10, unchanged after full write/read burn-in | Medium |
| Static SAS grown defects | Small nonzero count, unchanged | Medium |
| Old Other Error Count | Nonzero but not increasing | Low to medium |
| Old CRC/link errors | Nonzero but not increasing | Low to medium |
| Power-on hours | >40k hours | Low by itself |
| Power-on hours | >60k hours | Medium by itself |
| Temperature outlier | 5–8°C hotter than neighbors | Medium |
| Sustained temp | >45°C during heavy test | Medium |
| Sustained temp | >50°C | High concern |
| Slow drive | >20–30% worse sequential rate than same model peers | Medium |
| Latency outlier | 99th percentile latency 2× peer median under same load | Medium to high |
| SMART self-test aborts | Repeated aborts not caused by controller workload | Medium to high |

A drive made before 2020 could easily have 40k–50k+ hours if it ran 24/7. That does not automatically mean “bad,” but it does mean I would not mix it blindly into a critical array unless it passes a full-surface burn-in cleanly.

## Probably okay

| Signal | Interpretation |
| --- | --- |
| SMART passed, zero media errors, zero pending/uncorrectable, zero predictive failure |
| No new errors after full write/read + random stress |
| Stable temperature similar to neighbor drives |
| Similar throughput and latency to same model/firmware peers |
| No new controller events |
| No new PHY/link errors |
| Long self-test passes |

This is the only group I’d use for production data.

# 4. Understand the controller counters

On MegaRAID, pay attention to these fields:

## Media Error Count

This is the big one. It points toward real media trouble: unreadable sectors, remaps, failed reads, etc.

My rule:

text

0 = acceptable

>0 old/static = suspicious

increasing = reject

If a drive has nonzero media errors before you test it, I’d probably not use it in a critical array unless you are short on disks. If it gains even one media error during burn-in, reject it.

## Other Error Count

This is trickier. “Other” can be command aborts, link problems, resets, timeouts, enclosure/backplane issues, or drive firmware weirdness. It is not automatically a bad platter/surface.

My rule:

text

0 = ideal

1–5 static = watch

>10 static = investigate

increasing = reject or slot-swap test

If Other Error Count follows the drive to another bay, blame the drive.
If it stays with the slot, blame the bay, backplane, cable, expander lane, or power.

## Predictive Failure Count

Treat this as a hard reject. Seagate’s own guidance is that SMART errors are near-term failure predictions and are valid reasons to consider replacement. (Seagate.com)

## Shield Counter

On MegaRAID, drive shielding usually means the controller had to isolate/protect the array from a drive behaving badly or too slowly. One historical shield event is a warning. More than one, or any during acceptance testing, is a reject for production.

## PHY error counters

These are not necessarily drive failures. They may indicate:

text

Bad/backed-out carrier

Dirty connector

Bad slot

Bad backplane

Expander issue

SAS cable issue

Signal integrity problem

Drive port issue

The right test is a swap test:

text

Problem follows drive -> drive problem

Problem stays in slot -> chassis/backplane/path problem

Problem affects several adjacent slots -> backplane/expander/cable/power/cooling

# 5. Run a destructive pre-production burn-in

Assuming the data on these drives does not matter, I’d do this:

text

Phase A: baseline all counters

Phase B: long SMART/self-test

Phase C: full sequential write

Phase D: full sequential read/verify

Phase E: 12–24 hours random read/write stress

Phase F: final full sequential read

Phase G: collect all counters again and diff

For 8 TB HDDs, expect full-drive passes to take a long time. That is the point. You want infant-mortality and mishandled-used-drive failures to happen now, not during rebuild.

If the controller is in RAID personality and does not expose JBOD cleanly, you can create temporary single-disk RAID0 VDs for testing. If it supports JBOD, JBOD is cleaner for drive-level testing.

Do not run this against any mounted filesystem or any disk containing data you care about.

Example fio full-write pass against candidate raw block devices:

bash

fio --name=fullwrite \

  --filename=/dev/sdX \

  --rw=write \

  --bs=1M \

  --iodepth=16 \

  --ioengine=libaio \

  --direct=1 \

  --numjobs=1 \

  --group_reporting

Then full-read:

bash

fio --name=fullread \

  --filename=/dev/sdX \

  --rw=read \

  --bs=1M \

  --iodepth=16 \

  --ioengine=libaio \

  --direct=1 \

  --numjobs=1 \

  --group_reporting

For all candidate single-disk VDs, use /dev/disk/by-id/ paths so you do not accidentally hit the OS disks:

bash

ls -l /dev/disk/by-id/

Then make a drive list:

bash

cat > /root/test-disks.txt <<'EOF'

/dev/disk/by-id/scsi-...

/dev/disk/by-id/scsi-...

/dev/disk/by-id/scsi-...

EOF

Run one fio job per disk:

bash

while read d; do

  b=$(basename "$d")

  fio --name="write_$b" \

      --filename="$d" \

      --rw=write \

      --bs=1M \

      --iodepth=16 \

      --ioengine=libaio \

      --direct=1 \

      --numjobs=1 \

      --group_reporting \

      --output="/root/drive-audit/write_$b.log" &

done < /root/test-disks.txt

wait

Then read:

bash

while read d; do

  b=$(basename "$d")

  fio --name="read_$b" \

      --filename="$d" \

      --rw=read \

      --bs=1M \

      --iodepth=16 \

      --ioengine=libaio \

      --direct=1 \

      --numjobs=1 \

      --group_reporting \

      --output="/root/drive-audit/read_$b.log" &

done < /root/test-disks.txt

wait

Then random stress:

bash

while read d; do

  b=$(basename "$d")

  fio --name="rand_$b" \

      --filename="$d" \

      --rw=randrw \

      --rwmixread=70 \

      --bs=64k \

      --iodepth=32 \

      --ioengine=libaio \

      --direct=1 \

      --numjobs=1 \

      --runtime=24h \

      --time_based \

      --group_reporting \

      --output="/root/drive-audit/rand_$b.log" &

done < /root/test-disks.txt

wait

For HDDs, I care more about surface coverage and error deltas than synthetic peak numbers. Random stress is useful because it shakes out marginal mechanics, head positioning, timeouts, resets, and thermal behavior, but full sequential write/read is what actually touches the whole disk.

# 6. Monitor during the burn-in

In another terminal:

bash

watch -n 60 '

date

storcli /c0/eall/sall show | egrep "^[0-9]+:[0-9]+|EID|Drive"

echo

storcli /c0/eall/sall show phyerrorcounters 2>/dev/null | egrep "^[0-9]+:[0-9]+|Invalid|Loss|Phy|Error" || true

echo

dmesg -T | egrep -i "error|fail|reset|timeout|megaraid|sas|scsi|blk_update|I/O error" | tail -40

'

After each major phase:

bash

stamp=$(date +%F_%H%M)

mkdir -p /root/drive-audit/$stamp

storcli /c0/eall/sall show all > /root/drive-audit/$stamp/pd_all.txt

storcli /c0/eall/sall show phyerrorcounters > /root/drive-audit/$stamp/phy.txt 2>&1

storcli /c0 show events > /root/drive-audit/$stamp/events.txt

dmesg -T > /root/drive-audit/$stamp/dmesg.txt


      

for did in $(storcli /c0/eall/sall show | awk '/^[0-9]+:[0-9]+/ {print $3}' | sort -n); do

  smartctl -x -d megaraid,$did /dev/sda > /root/drive-audit/$stamp/smart_did_${did}.txt 2>&1

done

The important part is not just the final value. It is:

text

Before burn-in counter

After write counter

After read counter

After random stress counter

Final counter

Any bad counter that moves during the test is much more damning than an old static counter.

# 7. How I’d classify the 36 drives

After burn-in, put every drive into one of four buckets.

## Bucket 1 — production candidates

Requirements:

text

No SMART fail

No predictive failure

No media errors added

No uncorrectables

No pending sectors

No new grown defects

No resets/timeouts

No concerning dmesg events

No new PHY errors

Performance in normal peer range

Temperature normal for its bay

These are your array members.

## Bucket 2 — production hot spares

I’d be slightly stricter for hot spares than people usually are. A hot spare is not a junk drawer; it is the disk you will depend on during a degraded rebuild.

Good spare:

text

Clean burn-in

No new errors

Same capacity/sector format

Same or compatible interface

Reasonable power-on hours

No weird firmware mismatch

Do not use marginal disks as global hot spares for critical arrays. That’s how a tolerable single-drive failure becomes a rebuild failure.

## Bucket 3 — cold spare / lab / noncritical

Examples:

text

High power-on hours but clean

One or two old reallocated sectors, stable after destructive test

Old Other Error Count that does not move

Slightly slower but stable

Temperature mildly higher but explainable by bay position

These can be useful, but I would not place them in your main production array unless you are forced to.

## Bucket 4 — reject

Examples:

text

Any new media error

Any pending sector

Any uncorrectable

Predictive failure

Failed long test

Increasing grown defect list

Repeated timeouts

Repeated resets

Other Error Count increasing and following the drive

PHY errors following the drive

Drive disappears under load

Drive performs wildly worse than peers

Physically label these and remove them from the production candidate pool.

# 8. Things people often miss

## Sector format mismatches

Before building arrays, check:

text

512n

512e

4Kn

PI / T10 protection enabled or not

SED status

SAS vs SATA

firmware revision

Do not casually mix 512e and 4Kn in the same RAID group. Do not mix drives with odd PI/protection settings unless you know exactly what the controller and OS expect.

## Same model, different firmware

Same model with different firmware can behave differently under error recovery. If you have enough drives, group arrays by:

text

same model

same firmware

same sector size

similar age

similar performance

## Slow drives are failure risks

A drive does not need to throw hard media errors to be dangerous. A disk that repeatedly stalls, times out, or has very high tail latency can poison a hardware RAID group. It can make rebuilds crawl, trigger controller shielding, or cause array-level timeouts.

## Backplane issues masquerade as drive issues

With a 36-bay chassis, I would assume some percentage of weirdness may be:

text

carrier seating

slot connector

expander path

backplane lane

power delivery

fan/airflow imbalance

vibration coupling

If a problem appears, swap:

text

Drive A in bad slot -> good slot

Known-good drive -> suspicious slot

Then retest. You need to know whether you have a bad drive or a bad bay.

## Cooling under random write matters

A drive can look fine idle and cook during random write. Seagate specifically calls out high temperature as one of the SMART-monitored problem areas and notes that ventilation issues can cause SMART temperature warnings. (Seagate.com)

For dense 3.5" bays, I’d prefer:

text

<40°C excellent

40–45°C acceptable

45–50°C watch closely

>50°C fix airflow

>55°C unacceptable for sustained production if avoidable

## “SMART passed” is not enough

Backblaze’s data is a good reality check: the five SMART stats they used caught warning signs in many failed drives, but not all failed drives showed those warnings. (Backblaze) That means you need SMART plus controller logs plus burn-in behavior.

# 9. My acceptance policy for your Relion

Given your stated priorities — IOPS first, resiliency second, bandwidth third, capacity last — I’d be conservative:

text

1. Reject anything with predictive failure.

2. Reject anything that adds media errors during testing.

3. Reject anything with pending or uncorrectable sectors.

4. Reject anything with increasing grown defects.

5. Reject anything with repeated timeouts/resets.

6. Investigate any drive with Other Error Count >0.

7. Slot-swap any drive or bay with PHY errors.

8. Keep at least 2 clean global hot spares, preferably 3–4 if old disks are all you have.

9. Do not use marginal disks as hot spares.

10. Keep a written bay/serial/model/firmware/error baseline before production.

For inherited pre-2020 disks, I would not be surprised if some pass SMART but fail under full-surface testing. That is exactly why the burn-in matters. The right mindset is: make them prove they belong in the production pool.