From: https://chatgpt.com/c/69fcbfd8-a18c-83ea-8486-ae9ca43a32ab

# you asked: New-to-us

message time: 2026-05-07 12:37:48

New-to-us server (Relion xe4112 with an lsi 3108 and 36 drive bays). Some of the drives were made before 2020, I'm concerned about their health/how much abuse they've seen. Tell me how to investigate drive health, what to look out for (thesholds and confidence levels), how to identify marginal drives, etc.

# chatgpt response: I’ll treat

I’ll treat this like a professional intake/burn-in procedure: first establish the controller/drive visibility problem, then collect evidence, then run destructive/non-destructive tests, then classify drives by confidence rather than pretending SMART gives a clean pass/fail.

A key framing point: “SMART PASSED” is weak evidence. For old enterprise SAS disks behind MegaRAID, you want controller counters, the drive’s own SCSI log pages, and behavior under patrol/read/write stress all agreeing.

## Bottom line

For this server, I would not decide drive health from “SMART passed.” I’d build a per-slot evidence ledger, run a before/after burn-in, and classify drives by whether errors are present, new, trending, or slot/path-related.

With an LSI/Broadcom MegaRAID 3108, you need three views:

- MegaRAID controller view: media errors, other errors, predictive failure count, SMART alert, shield counters, link/PHY issues.

- Drive-native view: SAS/SCSI log pages or ATA SMART attributes via smartctl.

- Behavior under load: full-surface reads/writes, self-tests, performance outliers, resets, drops, timeouts.

smartctl supports ATA/SATA and SCSI/SAS SMART/self-test reporting, and it has explicit MegaRAID passthrough support using -d megaraid,N; the N value is the controller disk number, and you should validate it by matching serial numbers. (Arch Manual Pages)

# 1. First: inventory everything and preserve the baseline

Do this before creating arrays, clearing counters, initializing disks, or “fixing” anything.

bash

mkdir -p /root/drive-audit/$(date +%F)

cd /root/drive-audit/$(date +%F)


      

storcli /call show all > controller_all.txt

storcli /c0 show all > c0_all.txt

storcli /c0/eall/sall show > pd_summary.txt

storcli /c0/eall/sall show all > pd_all.txt

storcli /c0/eall/sall show all J > pd_all.json

storcli /c0/vall show all > vd_all.txt

storcli /c0/fall show all > foreign_all.txt

storcli /c0 show events > c0_events.txt

storcli /c0 show termlog type=contents > c0_termlog.txt

storcli /c0 show pdfailevents > c0_pdfailevents.txt


      

dmesg -T > dmesg_baseline.txt

journalctl -k --no-pager > kernel_baseline.txt

lsscsi -g > lsscsi_g.txt

lsblk -o NAME,TYPE,SIZE,MODEL,SERIAL,WWN,HCTL > lsblk.txt

Broadcom StorCLI documentation covers physical-drive display commands such as /c0/eall/sall show all, and it also has predictive failure event commands like /c0 show pdfailevents; those are exactly the controller-side counters you want preserved before testing. (Broadcom Docs)

Create a working table with at least:

| Field | Why it matters |
| --- | --- |
| Bay / physical slot | What you actually pull |
| EID:Slt | MegaRAID enclosure/slot identity |
| DID | Usually what smartctl -d megaraid,DID needs |
| Serial / WWN | Stable drive identity |
| Model / firmware | Look for mismatched firmware or bad batches |
| Power-on hours | Abuse / age proxy |
| Temp current / max if available | Cooling / abuse proxy |
| Controller media errors | Controller saw bad media |
| Controller other errors | Often path, cable, expander, reset, timeout |
| Predictive failure count | Serious |
| SMART alert flagged | Serious |
| SAS grown defect list | Reallocated grown defects |
| SAS read/write/verify uncorrected | Serious |
| Non-medium errors | Path/device/controller issue |
| Self-test status | Full-drive diagnostic result |
| Burn-in delta | Most important evidence |

# 2. Understand what matters on SAS drives

Do not expect old enterprise SAS drives to look like consumer SATA SMART output. SAS/SCSI does not usually expose the familiar ATA-style SMART attribute table. Seagate’s openSeaChest documentation explains that SCSI/SAS health reporting is largely through standardized log pages such as write error counters, read error counters, verify error counters, non-medium errors, and last-N error events, not just ATA SMART attributes. (GitHub)

For each drive, pull native data:

bash

# Pick one OS-visible MegaRAID device. Usually /dev/sda if you have a VD.

# Replace DID with the DID from storcli output. Validate by serial.

smartctl -x -d megaraid,DID /dev/sda > smart_DID.txt


      

# Also try FARM on Seagate drives if supported:

smartctl -l farm -d megaraid,DID /dev/sda > smart_DID_farm.txt

Smartmontools 7.4 added smartctl -l farm support for Seagate’s Field Access Reliability Metrics log for ATA and SCSI drives, which can expose better abuse/workload/environment history on supported Seagate drives. (GitHub) Seagate’s openSeaChest_SMART --showFARM also reports FARM data, including general drive info, workload statistics, error statistics, environment statistics, and reliability statistics, though availability varies by model and generation. (Ubuntu Manpages)

If smartctl does not map cleanly, try:

bash

smartctl --scan-open

smartctl -x -d megaraid,0 /dev/bus/0

smartctl -x -d megaraid,1 /dev/bus/0

Then match serial number against StorCLI. Do not assume DID, slot number, and bay number are the same.

# 3. Controller-side red flags

From:

bash

storcli /c0/eall/sall show all

Look for these fields:

text

Media Error Count

Other Error Count

Predictive Failure Count

S.M.A.R.T alert flagged by drive

Shield Counter

Drive Temperature

Firmware state

Link Speed

Connected Port

SN

WWN

Model Number

## Controller-side interpretation

| Signal | Meaning | My action |
| --- | --- | --- |
| Predictive Failure Count > 0 | Controller/drive has already crossed a predictive-failure threshold | Reject for production |
| S.M.A.R.T alert flagged by drive = Yes | Drive is reporting failure/prefailure | Reject |
| Media Error Count > 0 | Controller saw unrecoverable/serious media issue | Reject or quarantine unless clearly stale and no growth after full burn-in |
| Media Error Count increases during testing | Active surface/media failure | Reject with high confidence |
| Other Error Count > 0 | Often link/path/reset/transport, sometimes device | Investigate by slot-swap |
| Other Error Count increases on same bay with different disks | Backplane, expander, cable, controller, power issue | Fix chassis path, not just drive |
| Shield Counter > 0 | MegaRAID has had to protect/temporarily isolate a troubled drive | Treat as suspicious |
| Drive drops offline / foreign / bad / unconfigured bad | Obvious failure behavior | Reject |
| Repeated resets/timeouts in event log | Drive or path instability | Quarantine, then slot-swap test |

Important nuance: controller media errors and drive-native grown-defect counts do not always match. A disk may have remapped internally without the controller seeing it, or the controller may have recorded an error that did not result in a visible grown defect. So do not use only one side.

# 4. Drive-native red flags: SAS/SCSI

In smartctl -x -d megaraid,DID /dev/sda, look for:

text

SMART Health Status

Current Drive Temperature

Accumulated power on time

Elements in grown defect list

Error counter log

  read:

  write:

  verify:

Non-medium error count

Self-test log

Background scan results

## SAS/SCSI thresholds

| Signal | Confidence | Interpretation | Action |
| --- | --- | --- | --- |
| SMART Health Status: FAIL or failed informational exception | Very high | Drive says it is bad | Reject |
| Elements in grown defect list > 0 | Medium to high | Drive has remapped grown defects since manufacture | Quarantine; reject if critical or if count grows |
| Grown defect list increases during your testing | Very high | Active media deterioration | Reject |
| Read Total uncorrected errors > 0 | Very high | Drive could not recover read data | Reject |
| Write Total uncorrected errors > 0 | Very high | Write path/media failure | Reject |
| Verify Total uncorrected errors > 0 | High | Surface/verify failure | Reject |
| Non-medium error count increasing | Medium | Could be cable/backplane/expander/controller or drive electronics | Slot-swap isolate |
| Corrected errors climbing fast | Medium | ECC is saving you now; latency and future failure risk | Quarantine |
| Background scan found defects | High if recent/growing | Drive is finding bad media during internal scans | Quarantine/reject |
| Self-test read failure | Very high | Diagnostic hit unreadable media | Reject |
| Self-test abort due to host reset | Low alone | Could be controller/power/event | Retest and inspect logs |

For a critical old-drive file server, I would be harsh: any uncorrected SAS read/write/verify error is a reject. A small nonzero grown defect list is not mathematically the same as immediate death, but for your use case it is a marginal drive, not a “good” drive.

# 5. Drive-native red flags: SATA/ATA, if any are SATA behind the 3108

Backblaze’s large-scale drive-health work highlights five SMART attributes they used as failure-investigation triggers: 5 Reallocated Sectors, 187 Reported Uncorrectable, 188 Command Timeout, 197 Current Pending Sector, and 198 Uncorrectable Sector Count. They also note that no single SMART stat catches all failures; in their dataset, 76.7% of failed drives had at least one of those five attributes greater than zero, while 23.3% of failed drives showed no warning in those tracked stats. (Backblaze)

Use these as investigation triggers, not magic truth.

| ATA SMART attribute | Action |
| --- | --- |
| 5 Reallocated_Sector_Ct > 0 | Marginal; reject if growing or critical-use |
| 187 Reported_Uncorrect > 0 | Reject |
| 188 Command_Timeout > 0 | Quarantine; often path/power/drive hang |
| 197 Current_Pending_Sector > 0 | Reject for production unless destructive rewrite clears it and no recurrence; for your use case, still suspicious |
| 198 Offline_Uncorrectable > 0 | Reject |
| 199 UDMA_CRC_Error_Count > 0 | Usually cable/backplane/path; if increasing, fix path |
| Raw Seagate Raw_Read_Error_Rate huge value | Usually not directly meaningful raw; do not panic from raw value alone |
| SMART overall passed | Weak evidence only |

Backblaze’s point about time distribution matters: a drive that jumps from 0 to 20 uncorrectables in one day is much scarier than a drive with a small stale count accumulated years ago. (Backblaze) That is exactly why your before/after delta matters more than a single snapshot.

# 6. Age, abuse, and confidence

For pre-2020 drives, power-on hours matter, but age alone is not a diagnostic.

Approximate interpretation:

| Power-on hours | Calendar equivalent | Interpretation |
| --- | --- | --- |
| < 10,000 | <1.2 years 24/7 | Light use or reset/replacement history |
| 10,000–30,000 | 1.1–3.4 years | Normal used enterprise |
| 30,000–45,000 | 3.4–5.1 years | Aged but plausible |
| 45,000–60,000 | 5.1–6.8 years | Old; expect higher failure probability |
| > 60,000 | >6.8 years | Retirement-grade for critical infrastructure |

What I’d treat as abuse evidence:

| Evidence | Confidence |
| --- | --- |
| High power-on hours | Medium |
| High host reads/writes / workload stats from FARM | Medium to high |
| High max lifetime temperature or many over-temp events | Medium to high |
| Grown defects | Medium to high |
| Uncorrected errors | Very high |
| High power cycles alone | Low to medium |
| Old manufacture date but low POH | Low; shelf-aged, not necessarily abused |
| Controller counters already nonzero | Medium unless they grow during your test |
| Errors across multiple drives in same bay group | High for chassis/path issue |

A drive with 55k hours and zero errors may still work. A drive with 8k hours and new uncorrected errors is trash.

# 7. Burn-in procedure I’d actually use

## Phase A: baseline capture

Already covered above. Save everything.

## Phase B: SMART short tests

Run on every physical drive:

bash

smartctl -t short -d megaraid,DID /dev/sda

Then later collect:

bash

smartctl -x -d megaraid,DID /dev/sda > smart_DID_after_short.txt

Failures here are immediate rejects. Passing short tests does not prove much.

## Phase C: SMART long / extended tests

For each drive:

bash

smartctl -t long -d megaraid,DID /dev/sda

Then:

bash

smartctl -x -d megaraid,DID /dev/sda > smart_DID_after_long.txt

Reject anything with:

text

Completed: read failure

Completed: write failure

Completed: unknown failure

Total uncorrected errors increased

Elements in grown defect list increased

## Phase D: full-surface read

If the disks are already in VDs, a full read of the VD is better than nothing. If you can expose drives as JBOD or temporary single-disk RAID0 devices before production, that is better.

For an OS-visible test device:

bash

fio --name=readpass \

  --filename=/dev/sdX \

  --rw=read \

  --bs=1M \

  --iodepth=32 \

  --direct=1 \

  --numjobs=1 \

  --ioengine=libaio \

  --group_reporting

Watch during the run:

bash

journalctl -kf

dmesg -Tw

iostat -xm 5

watch -n 60 'storcli /c0/eall/sall show all | egrep -i "Drive /c|Media Error Count|Other Error Count|Predictive Failure Count|S.M.A.R.T alert|Shield Counter|Drive Temperature"'

Any new media errors, resets, drops, or uncorrected errors are rejects.

## Phase E: destructive write/read/verify burn-in

Only do this before production, on test devices with no data.

For fio verify:

bash

fio --name=verifyburn \

  --filename=/dev/sdX \

  --rw=write \

  --bs=1M \

  --iodepth=16 \

  --direct=1 \

  --ioengine=libaio \

  --verify=crc32c \

  --do_verify=1 \

  --verify_fatal=1 \

  --size=100% \

  --group_reporting

Alternative with badblocks, destructive:

bash

badblocks -wsv -b 4096 -c 65536 /dev/sdX

For 8 TB disks, use -b 4096; otherwise older assumptions around block count can bite you on large devices. I generally prefer fio for a server burn-in because it gives better performance visibility and integrates better with concurrent load testing.

## Phase F: all-drive concurrent load

This is where marginal old systems show themselves. One disk at a time finds bad platters. All disks at once finds:

- expander issues

- backplane issues

- cooling problems

- PSU sag

- vibration sensitivity

- controller resets

- cachevault/BBU problems

- bays or cable groups with correlated errors

Run read workload across all candidate devices. If doing destructive testing, write/verify all candidate devices concurrently.

Then compare:

bash

storcli /c0/eall/sall show all > pd_all_after_burnin.txt

storcli /c0 show events > c0_events_after_burnin.txt

storcli /c0 show termlog type=contents > c0_termlog_after_burnin.txt

dmesg -T > dmesg_after_burnin.txt

journalctl -k --no-pager > kernel_after_burnin.txt

Then rerun every smartctl -x.

The delta is the important part.

# 8. How I would classify the drives

## Class A — production-good

Requirements:

- No controller media errors.

- No predictive failure count.

- No SMART alert.

- No uncorrected read/write/verify errors.

- No new grown defects.

- Self-tests pass.

- Full-surface read passes.

- Destructive write/read/verify passes, if performed.

- No resets, drops, or timeouts.

- Performance is within normal range for the same model.

- Temperature behavior is normal under all-drive load.

These are candidates for your actual arrays.

## Class B — usable but not trusted

Examples:

- High power-on hours but clean tests.

- Small stale grown defect list, no growth.

- Small stale controller media count, no growth after full burn-in.

- Mild corrected-error history but no uncorrected errors.

- Older firmware but otherwise clean.

- Slightly higher temps due to bay position.

I would use these only as:

- cold spares,

- temporary migration disks,

- scratch/test storage,

- non-critical backup targets,

- never as the first-choice members in a critical RAID set.

## Class C — marginal / quarantine

Examples:

- Any nonzero media error count that you cannot prove is stale.

- Any grown defects plus high age.

- Corrected errors increasing under test.

- Intermittent timeout/reset behavior.

- Performance outlier.

- Non-medium errors that may follow the drive after slot swap.

- Self-test aborted repeatedly without clear host cause.

These should not go into a critical array.

## Class D — reject

Immediate reject:

- SMART alert flagged.

- Predictive failure count greater than zero.

- Any new media error during testing.

- Any uncorrected read/write/verify error.

- Current pending sectors on SATA.

- Offline uncorrectable on SATA.

- Self-test read/write failure.

- Drive drops offline.

- Grown defect list increases during burn-in.

- Verify mismatch.

- Repeated command timeouts.

- Fails in multiple bays.

- Causes controller resets or bus instability.

Do not rationalize these into service because the array has parity. Old arrays fail by correlated marginal devices, not just by one obvious dead disk.

# 9. Slot-swap method for ambiguous failures

When you see Other Error Count, non-medium errors, PHY errors, CRC errors, or resets, do this:

- Record drive serial, DID, EID:Slt, bay.

- Move the suspicious drive to a known-good bay.

- Move a known-good drive to the suspicious bay.

- Rerun read/write stress.

- Compare where the errors go.

Interpretation:

| Result | Likely cause |
| --- | --- |
| Errors follow the drive | Drive electronics/media/interface |
| Errors stay with bay | Backplane/slot/cable/expander |
| Errors affect a group of bays | Mini-SAS cable, expander lane, power rail, airflow, vibration |
| Errors appear only under all-drive load | Power, cooling, vibration, controller, expander |
| Errors appear only during writes | Drive media, cache policy, power-loss/capacitor issue, controller cache path |

For a 36-bay chassis, group patterns matter. If bays 0–5, 6–11, or an entire row show similar symptoms, suspect the chassis path before blaming six drives independently.

# 10. Things people often overlook

## “Passed SMART” is not a clean bill of health

SMART is useful, but it is not complete. Backblaze’s own analysis found a meaningful fraction of failed drives without warning in the tracked SMART stats. (Backblaze) Use SMART as evidence, not as a binary oracle.

## Old same-batch drives fail together

If all 30–36 drives came from the same environment, same age, same workload, and same chassis, your risk is correlated. Two hot spares help availability, but they do not solve the problem of a whole batch entering wear-out together.

## RAID rebuilds are drive-health tests

A rebuild reads every surviving member hard while writing the replacement. Marginal drives often fail during rebuild. That is why you want burn-in before array creation, and patrol reads/consistency checks after.

## Controller logs can be stale

A drive may show Media Error Count = 1 from some ancient event. That is still suspicious, but the critical question is whether it grows during your controlled tests.

## Path problems masquerade as drive problems

Non-medium errors, CRC errors, link resets, and timeouts are often backplane/cable/expander/power issues. Do not keep replacing disks in the same bad slot.

## Performance outliers matter

A disk with no logged errors but 40% lower sequential throughput, erratic latency, or repeated long stalls is marginal. For RAID, a slow disk can hurt the whole virtual drive and may be approaching failure.

# 11. Suggested production monitoring after you deploy

At minimum:

bash

storcli /c0/eall/sall show all

storcli /c0/vall show all

storcli /c0 show events

storcli /c0 show pdfailevents

Alert on:

- predictive failure count > 0

- SMART alert yes

- media error count increase

- other error count increase

- shield counter increase

- drive state not online / hotspare / good as expected

- virtual drive degraded

- patrol read errors

- consistency check errors

- temperature outliers

- controller cachevault/BBU not optimal

For smartd, configure MegaRAID passthrough entries per DID, but remember to regenerate them after drive replacement because DIDs can change.

Example style:

text

/dev/sda -d megaraid,0 -a -o on -S on -s (S/../.././02|L/../../6/03) -m root

/dev/sda -d megaraid,1 -a -o on -S on -s (S/../.././02|L/../../6/03) -m root

Validate serial mapping before trusting alerts.

# 12. My practical acceptance policy for your server

Given “old but critical infrastructure,” I’d use this policy:

Reject immediately:

- predictive failure

- SMART alert

- any uncorrected error

- any new media error

- any new grown defect

- self-test failure

- drive drop/reset under load

- current pending/offline uncorrectable

- verify mismatch

Quarantine:

- any existing media error

- any grown defect list > 0

- high corrected-error rate

- non-medium errors until slot-swapped

- large performance outlier

- very high POH combined with any other blemish

Accept:

- clean baseline

- clean long self-test

- clean full-surface read

- clean destructive verify if performed

- no event log noise

- no path errors

- normal temps

- normal performance

For a 36-drive old-disk pool, I would rather run with fewer known-good drives and more cold spares than build a maximum-capacity array out of marginal disks. With old used drives, the dangerous ones are not the dead ones. The dangerous ones are the ones that are “mostly fine” until the first rebuild.