From: https://gemini.google.com/app/599c0298f26c186a

# you asked: New-to-us

message time: 2026-05-07 12:32:06

New-to-us server (Relion xe4112 with an lsi 3108 and 36 drive bays). Some of the drives were made before 2020, I'm concerned about their health/how much abuse they've seen. Tell me how to investigate drive health, what to look out for (thesholds and confidence levels), how to identify marginal drives, etc.

# gemini response: ### TL;DR

### TL;DR

Use storcli for high-level RAID controller status and smartctl (with the -d megaraid,N flag) for raw health data. Focus on Reallocated Sectors, Current Pending Sectors, and Media Error Counts. Drives from 2020 or earlier likely have 40,000–50,000+ power-on hours; if reallocated sectors are non-zero and increasing, swap them immediately.

## 1. Hardware-Level Assessment (`storcli`)

The LSI 3108 (MegaRAID) controller abstracts physical drives. You need storcli to see what the controller thinks of them.

Check all drives quickly:

bash

storcli /c0 /eall /sall show all | grep -E "S.M.A.R.T|Predictive|Media Error|Other Error"

- Predictive Failure Count: If this is anything other than 0, the drive’s internal firmware has already flagged itself for imminent death.

- Media Error Count: Indicates physical platter/NAND issues. Small numbers might be corrected, but a growing count indicates a "marginal" drive.

- Other Error Count: Often points to communication issues (cables, backplane, or controller timeouts).

## 2. Deep Dive with SMART Data

To get the actual "abuse" metrics, you must bypass the RAID abstraction using smartmontools. Note that you must target the specific device ID behind the controller.

Examine a specific drive (where 0 is the device ID):

bash

smartctl -a -d megaraid,0 /dev/sda

### Key Attributes to Monitor

| Attribute | Threshold/Red Flag | Significance |
| --- | --- | --- |
| Reallocated Sector Count | > 0 (and increasing) | The drive has "swapped" a bad sector for a spare. Once this starts climbing, failure is exponential. |
| Current Pending Sector | > 0 | Sectors waiting to be remapped. Usually indicates a "hang" or "hiccup" during read/write. |
| UDMA CRC Error Count | > 0 | Usually not a drive failure, but a bad backplane, cable, or loose connection in one of those 36 bays. |
| Power-On Hours | > 45,000 | For enterprise SAS/SATA, this is the start of the "yellow zone." For consumer drives, this is "borrowed time." |
| Wear Range Delta (SSD) | > 5-10% | Significant if you have a mix of drives; indicates uneven wear across the array. |

## 3. Identifying "Marginal" Drives (The Silent Killers)

A drive can have a "Healthy" SMART status but still be "marginal"—meaning it’s slow enough to degrade the entire array's performance without hard-failing.

### Latency Checks

Use iostat while the array is under load to find drives with high wait times (await) or service times (svctm).

bash

iostat -xny 5

Look for a single drive where %util is 100% while others are at 30%, or where latency is consistently >20ms compared to its peers.

### The "Slow Read" Test

If the drives are in a JBOD or individual RAID 0s, you can test sequential read consistency:

bash

dd if=/dev/sdX of=/dev/null bs=1M count=1024 iflag=direct

A marginal drive will often show "stuttering" throughput—dropping from 200MB/s to 10MB/s and back—which indicates the firmware is struggling with error correction.

## 4. Confidence Levels & Strategy

- High Confidence Replacement: Any drive with Predictive Failure = Yes or Current Pending Sectors > 0. Don't wait; these will likely drop during a RAID rebuild (the "rebuild stress" often kills marginal drives).

- Medium Confidence (Watchlist): Drives with high power-on hours but 0 errors. These are "reliable" but old. If you have spares, replace the oldest ones first.

- The 36-Bay Trap: In high-density chassis like the xe4112, heat is the enemy of the middle-row drives. Pay special attention to the temperature readings in the smartctl output. If a drive is consistently 5–10°C hotter than its neighbors, its lifespan will be significantly shorter.

Pro-Tip: Since this is a "new-to-you" server, run a Badblocks non-destructive read test on the entire array. It will force the controller to touch every sector and will likely "trip" any marginal drives that were sitting idle.

bash

badblocks -nsv /dev/sdX

How many of the 36 bays are currently populated, and are you planning on running ZFS or a hardware RAID 5/6/10?